## 630E Pro

## USER'S MANUAL

M/B For Socket 370 Processor

Ver. 6.0

TABLE OF CONTENT

CHAPTER 1  INTRODUCTION OF 630E Pro MOTHERBOARD

1-1	FEATURE OF MOTHERBOARD	1

1-2	SPECIFICATION	2

1-3	PERFORMANCE LIST	3

1-4	LAYOUT DIAGRAM & JUMPER SETTING	4

CHAPTER 2  HARDWARE INSTALLATION

2-1	HARDWARE INSTALLATION STEPS	6

2-2	CHECKING MOTHERBOARD'S JUMPER SETTING	6

2-3	INSTALL CPU	8

2-4	INSTALL MEMORY	9

2-5	EXPANSION CARDS	10

2-5-1   PROCEDURE FOR EXPANSION CARD INSTALLATION	10

2-5-2   ASSIGNING IRQ FOR EXPANSION CARD	10

2-5-3   INTERRUPT REQUEST TABLE FOR THIS MOTHERBOARD	11

2-6	CONNECTORS, HEADERS	11

2-6-1   CONNECTORS	11

2-6-2   HEADERS	14

2-7	STARTING UP YOUR COMPUTER	17

CHAPTER 3  INTRODUCING BIOS

3-1	ENTERING SETUP	18

3-2	GETTING HELP	19

3-3	THE MAIN MENU	19

3-4	STANDARD CMOS FEATURES	20

3-5	ADVANCED BIOS FEATURES	22

3-6	ADVANCED CHIPSET FEATURES	24

3-6-1   ADVANCED DRAM CONTROL	25

3-7	INTEGRATED PERIPHERALS	26

3-7-1   ONCHIP IDE FUNCTION	27

3-7-2   ONCHIP DEVICE FUNCTION	28

3-7-3   WINBOND SUPERIO DEVICE	29

3-8	POWER MANAGEMENT SETUP	30

3-8-1   PM WAKE UP EVENTS	31

3-9	PNP/PCI CONFIGURATION SETUP	32

3-9-1   IRQ RESOURCES	33

3-10	PC HEALTH STATUS	34

3-11	FREQUENCY/VOLTAGE CONTROL 	34

3-12	LOAD STANDARD/OPTIMIZED DEFAULTS	35

3-13	SET SUPERVISOR/USER PASSWORD	36

CHAPTER 4  DRIVER & FREE PROGRAM INSTALLATION

4-1 	DRIVER INSTALLATION	37

4-2 	BIOS INSTALLATION UTILITY & BIOS UPDATE	38

用户简易安装手册	39

## Chapter 1

Introduction of 630E PRO Motherboard

# 1-1	 Feature of motherboard

The 630E PRO motherboard is designed for Intel’s new generation Pentium processors, which utilize the Socket 370 design supports PentiumIII/Celeron FC-PGA/PPGA processors, and the memory size expandable to 1GB.

This motherboard use the newest SiS 630 chipset provides a high performance/low cost solution for socket 370 series CPUs based system, by integrating a high performance North Bridge, advanced hardware 2D/3D GUI engine and Super South  Bridge. Moreover, by integrating the Ultra AGP technology and advanced 128-bit graphic display interface, SiS 630 delivers AGP 4X performance and up to 2GB/s memory bandwidth, and provides powerful hardware decoding DVD accelerator to improve the DVD playback performance.  On-board VGA memory can select from 2Mb to 64Mb.

This motherboard provides 10/10Mb Fast Ethernet interface for Office requirement and offers AC’97 compliant interface that comprises digital audio engine with 3D-hardware accelerator, on-chip sample rate converter. This motherboard also provides dual USB host controller with four USB Ports that deliver better connectivity and 2x12Mb bandwidth.  The built-in Fast PCI IDE controller supports Ultra DMA 33/66 function up to 66MB/s for data transfer rate.  In addition, 630E PRO provides hardware monitoring function that will monitor and protect your computer.

This motherboard provides high performance & meets future specification demand.  It is really the best choice for your computer.

# 1-2	 Specification

| Spec | Description |
| --- | --- |
| Design | - Micro ATX form factor 4 layers PCB size: 24.5x19.0cm |
| Chipset | - SiS 630 Chipset |
| Clock Generator | - Support 66/100/133MHz system Bus Clock (CPU Bus Clock) Support 100/133 MHz system memory clock Support 33MHz PCI Bus clock |
| CPU Socket | - Support Pentium III 5001.33GHz processor - Support Celeron 300~1.4GHz processor - Support 66, 100 and 133MHz CPU Bus clock - Reserves support for future Intel Pentium III processors |
| Memory Socket | - 168-pin DIMM socket x2 - PC-100/PC-133 SDRAM - Expandable to 1GB - Support 3.3V SDRAM DIMM |
| Expansion Slot | - 32-bit PCI slot x3, 2.2 Specification Compliant - AMR slot x1 |
| Integrate VGA | - 3D graphic acceleration - VGA Memory Selectable by BIOS from 2MB to 64MB |
| Integrate IDE | - 2 channel of Bus Master IDE port supporting ULTRA DMA 33/66MB/s mode devices |
| Integrate LAN | - Fast Ethernet Controller 10/100 Mbps |
| Audio | - AC’97 Digital Audio controller integrated - AC’97 Audio CODEC on board - Audio driver and utility included |
| BIOS | - Award 2MB Flash ROM |
| Multi I/O | - PS/2 keyboard and PS/2 mouse connectors - Floppy disk drive connector x1 - Parallel port x1 - Serial port x1, (1 port Optional) - USB connector x2 - USB headers x2 (connecting cable option) - Audio connector (Line-in, Line-out & Game Port) |

# 1-3	 Performance List

The following performance data list is the testing result of some popular benchmark testing programs. These data are just referred by users, and there is no responsibility for different testing data values gotten by users (the different Hardware & Software configuration will result in different benchmark testing results.)

CPU:			    Intel PIII 866MHz FC-PGA package

DRAM:			    128M SDRAM x2 (Hyundai GM 72V66841ET75)

VGA Expansion Card: On Board VGA (Driver V1.05)

Hard Disk Driver: 	    Quantum Fireball KX20A11

BIOS:			    Award Optimal default

OS:				    Win 98SE

Performance Test Report

|  | Share 8M | Share 16M | Share 32M |
| --- | --- | --- | --- |
| ## 3D Mark 99 | 1908 | 1918 | 1912 |
| ## 3D Mark 2000 | 900 | 918 | 916 |
| 3D Winbench 99 V1.2 | 513 | 525 | 446 |
| ## 3D Winbench 2000 | 15.7 | 20.7 | 23 |
| ## Final Reality | 5.23 | 5.25 | 5.20 |
| Winstone 99 V1.3 | 29.5 | 29.5 | 29.6 |
| Winstone 2000 | 32.3 | 33 | 31.8 |
| ##### Winbench 99 : |  |  |  |
| ## CPU Mark 99 | 67.1 | 68.1 | 68.5 |
| ## FPU Winmark 99 | 4390 | 4390 | 4380 |
| ## Business Disk Winmark99 | 4740 | 4550 | 4680 |
| ## Hi-end Disk Winmark99 | 17100 | 16800 | 16300 |
| ## Business Graphic Winmark | 217 | 216 | 217 |
| ## Hi-end Graphic Winmark | 762 | 767 | 767 |
| ##### SYS Mark 2000: SISMark 2000 Rating (Internet Content Creation / Office Productivity) |  |  |  |
| ## Suites | 156 | 157 | 157 |
| ## Offical | 154 | 155 | 156 |
| ### SISOFT Sandra 2000: |  |  |  |
| ## CPU MIPS | 2262 | 2262 | 2262 |
| ## FPU MFLOPS | 1120 | 1120 | 1120 |
| #### CPU / Memory MB/S | 234 | 236 | 235 |
| ## FPU / Memory MB/S | 255 | 257 | 256 |
| ### QUAKE3: |  |  |  |
| DEMO1 FPS | 22.0 | 22.1 | 22.2 |
| DEMO2 FPS | 21.3 | 21.6 | 21.6 |

# 1-4	 Layout Diagram & Jumper Setting

'

Jumpers

| Jumper | Name | Description |
| --- | --- | --- |
| JP7 | Only For PIII or Celeron 1.5V CPU | 1x2-pin Block |
| JP10, JP1, JP11 | CPU Type Setting | 3x3-pin Block |
|  | Celeron 300~500MHz | JP10, JP1, JP11 (2-3) |
|  | PIII / C3 / Celeron 500 ~ 1.4G | JP10 (1-2) / JP1, JP11 (open) |
| JP3 | CMOS RAM Clear | 1x3-pin Block |

## Connectors

| Connector | Name | Description |
| --- | --- | --- |
| ATX | ATX Power Connector | 20-pin Block |
| KB/MOUSE | PS/2 Mouse & PS/2 Keyboard Connector | 6-pin Female |
| USB | USB Port Connector | 2x5-pin Connector |
| LAN | LAN Port Connector | RJ45 Connector |
| PRINT | Parallel Port Connector | 25-pin Female |
| VGA | VGA Port Connector | 15-pin Female |
| GAME | Audio/Game Connector | 3 phone jack + 15-pin Connector |
| COM1 | Serial Port COM1 Connector | 10-pin Connector |
| FDD | Floppy Driver Connector | 34-pin Block |
| IDE1 | Primary IDE Connector | 40-pin Block |
| IDE2 | Secondary IDE Connector | 40-pin Block |

## Headers

| Header | Name | Description |
| --- | --- | --- |
| COM2 | COM2 Headers (Optional) | 10-pin Block |
| USB2 | USB Port Headers | 10-pin Block |
| IDE LED | IDE activity LED | 2-pin Block |
| RESET | Reset switch lead | 2-pin Block |
| KB LOCK | Keyboard lock switch | 2-pin Block |
| SPKR | Speaker connector | 4-pin Block |
| PWR LED | Power LED | 2-pin Block |
| PWR BTN | Power switch | 2-pin Block |
| WOL | Wake On-LAN Headers | 3-pin Block |
| CPUFAN SYSFAN | FAN Speed Headers | 3-pin Block |
| IR, CIR | IR infrared module Headers | 10-pin Block |
| CDIN1, CDIN2 | CD Audio-In Headers | 4-pin Block |
| AUX-IN | AUX-IN Audio Input Header | 4-pin Block |

## Expansion Sockets

| Socket/Slot | Name | Description |
| --- | --- | --- |
| ZIF Socket 370 | CPU Socket | 370-pin FC-PGA/PPGA CPU Socket |
| DIMM1, DIMM2 | DIMM Module Socket | 168-pin DIMM SDRAM Module Expansion Socket |
| PCI1, PCI2, PCI3 | PCI Slot | 32-bit PCI Local Bus Expansion slots |
| AMR | AMR Slot | Modem Riser Card slot |

## Chapter 2

Hardware installation

# 2-1	 Hardware installation Steps

Before using your computer, you had better complete the following steps:

- Check motherboard setting

- Install CPU

- Install Memory

- Install Expansion cards

- Connect Ribbon cables, Panel wires, and power supply

- Setup BIOS

- Install software driver & utility

# 2-2	 Checking Motherboard’s Jumper Setting

- Hardware CPU & SDRAM Clock Setting: JP2

The motherboard’s CPU & SDRAM memory clock can be adjusted through jumper JP2.

** If want to setting “CPU Freq” then run BIOS Jumpless function.

| CPU/SDRAM (MHz) | JP2 |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1-2 | 3-4 | 5-6 | 7-8 |
| 66/66 | OFF | OFF | OFF | ON |
| 66/100 | OFF | OFF | OFF | OFF |
| 100/100 | ON | OFF | OFF | OFF |
| 100/133 | ON | OFF | ON | OFF |
| 133/100 | ON | ON | OFF | OFF |
| 133/133 | ON | ON | ON | OFF |

- CPU Type setting:  JP7, JP10, JP1, JP11

This motherboard supports all socket 370 processors such as Pentium III, Celeron I/II/III. Please refer to the following diagram to set the CPU type.

** If CPU Voltage is 1.5V then JP7 setting “ON”

| For other CPU | For PIII or Celeron CPU with 1.5V Vcore |
| --- | --- |
| JP7 (Open / OFF) | JP7 (Close / ON) |

JP10, JP1, JP11 Setting Table

| For CPU 300~500MHz | For Other CPU |
| --- | --- |

- CMOS RAM Clear (3-pin): JP3

A battery must be used to retain the motherboard configuration in CMOS RAM short 2-3 pins of JP6 to store the CMOS data.

Note:	 You can clear CMOS by shorting 1-2 pin, while the system is off. Then return to 2-3pin position. Avoid clearing the CMOS while the system is on, it will damage the motherboad. Always unplug the power cord from the wall socket.

2-3 Install CPU

This motherboard Provides a ZIF socket 370.  The CPU that comes with the motherboard should have a cooling FAN attached to prevent overheating. If this is not the case, then purchase a correct cooling FAN before you turn on your system.

WARNING!	Be sure that there is sufficient air circulation across the processor’s heatsink and CPU cooling FAN is working correctly, otherwise it may cause the processor and motherboard overheat and damage, you may install an auxiliary cooling FAN, if necessary.

To install a CPU, first turn off your system and remove its cover.  Locate the ZIF socket and open it by first pulling the level sideways away from the socket then upward to a 90-degree angle.  Insert the CPU with the correct orientation as shown below.  The notched corner should point toward the end of the level.  Because the CPU has a corner pin for two of the four corners, the CPU will only fit in the orientation as shown.

When you put the CPU into the ZIF socket.  No force require to insert of the CPU, then press the level to Locate position slightly without any extra force.

# 2-4	 Install Memory

This motherboard provides two 168-pin DUAL INLINE MEMORY MODULES (DIMM) sites for memory expansion available from minimum memory size of 16MB to maximum memory size of 1GB SDRAM.

Valid Memory Configurations

| Bank | 168-Pin DIMM |  | Total Memory |
| --- | --- | --- | --- |
| Bank 0, 1 (DIMM1) | SDRAM 32, 64, 128, 256, 512MB | X 1 | 16MB512MB |
| Bank 2, 3 (DIMM2) | SDRAM 32, 64, 128, 256, 512MB | X 1 | 16MB512MB |
| Total | System Memory (Max. 1GB) |  | 16MB1GB |

Generally, installing SDRAM modules to your motherboard is very easy, you can refer to figure 2-4 to see what a 168-Pin PC100 & PC133 SDRAM module looks like.

|  | Figure 2-4 |
| --- | --- |

NOTE!	When you install DIMM module fully into the DIMM socket, the eject tab should be locked into the DIMM module very firmly and fit into its indention on both sides.

WARNING!	For the SDRAM CLOCK is set at 133MHz, use only PC133-compliant DIMMs. When this motherboard operate at 133Mhz, most system will not even boot if non-compliant modules are used because of the strict timing issues, if your DIMM are not PC133-compliant, set the SDRAM clock to 100MHz to ensure system’s stability.

# 2-5	 Expansion Cards

WARNING!	Turn off your power when adding or removing expansion cards or other system components.  Failure to do so may cause severe damage to both your motherboard and expansion cards.

# 2-5-1  Procedure For Expansion Card Installation

- Read the documentation for your expansion card and make any necessary hardware or software setting for your expansion card such as jumpers.

- Remove your computer’s cover and the bracket plate on the slot you intend to use.

- Align the card’s connectors and press firmly.

- Secure the card on the slot with the screen you remove above.

- Replace the computer system’s cover.

- Set up the BIOS if necessary.

- Install the necessary software driver for your expansion card.

# 2-5-2  Assigning IRQs For Expansion Card

Some expansion cards need an IRQ to operate.  Generally, an IRQ must exclusively assign to one use.  In a standard design, there are 16 IRQs available but most of them are already in use.

Standard Interrupt Assignments

| IRQ | Priority | Standard function |
| --- | --- | --- |
| 0 | 1 | System Timer |
| 1 | 2 | Keyboard Controller |
| 2 | N/A | Programmable Interrupt |
| 3 * | 11 | Communications Port (COM2) |
| 4 * | 12 | Communications Port (COM1) |
| 5 * | 13 | Sound Card (sometimes LPT2) |
| 6 | 14 | Floppy Disk Controller |
| 7 * | 15 | Printer Port (LPT1) |
| 8 | 3 | System CMOS/Real Time Clock |
| 9 * | 4 | ACPI Mode when enabled |
| 10 * | 5 | IRQ Holder for PCI Steering |
| 11 * | 6 | IRQ Holder for PCI Steering |
| 12 * | 7 | PS/2 Compatible Mouse Port |
| 13 | 8 | Numeric Data Processor |
| 14 * | 9 | Primary IDE Channel |
| 15 * | 10 | Secondary IDE Channel |

※These IRQs are usually available for ISA or PCI devices.

# 2-5-3  Interrupt Request Table For This Motherboard

Interrupt request are shared as shown the table below:

|  | INT A | INT B | INT C | INT D |
| --- | --- | --- | --- | --- |
| PCI slot 1 |  | Shared |  |  |
| PCI slot 2 |  |  | Shared |  |
| PCI slot 3 |  |  |  | Shared |
| Onboard VGA |  |  |  |  |
| AC97/MC97 |  | Shared |  |  |
| Onboard USB |  |  |  | Shared |
| Onboard USB 2 |  |  |  | Shared |
| LAN |  |  | Shared |  |

IMPORTANT!	If using PCI cards on shared slots, make sure that the drivers support “Shared IRQ” or that the cards don’t need IRQ assignments. Conflicts will arise between the two PCI groups that will make the system unstable or cards inoperable.

# 2-6	 Connectors, Headers

# 2-6-1  Connectors

(1)	Power Connector:  ATX (20-pin block)

ATX Power Supply connector.  This is a new defined 20-pins connector that usually comes with ATX case. The ATX Power Supply allows to use soft power on momentary switch that connect from the front panel switch to 2-pins Power On jumper pole on the motherboard.  When the power switch on the back of the ATX power supply turned on, the full power will not come into the system board until the front panel switch is momentarily pressed.  Press this switch again will turn off the power to the system board.

(2)	PS/2 Mouse & PS/2 Keyboard Connector: KB/MOUSE

The connectors for PS/2 keyboard and PS/2 Mouse.

(3)	USB Port connector: USB

The connectors are 4-pins connector that connect USB devices to the system board.

(4)	LAN Port connector: LAN

This connector is standard RJ45 connector for Network connector.

(5)	Parallel Port Connector (25-pin female): PRINT

Parallel Port connector is a 25-pin D-Subminiature Receptacle connector.  The On-board Parallel Port can be disabled through the BIOS SETUP.  Please refer to Chapter 3 “INTEGRATED PERIPHERALS SETUP” section for more detail information.

(6)	VGA Connector (15-pin female): VGA

(7)	Audio and Game Connector: GAME

This connector is a jack for LINE-OUT, LINE-IN, MIC and a 15-pin D-Subminiature Receptacle Connector for joystick/MIDI Device.

Line-out: 	Audio output to speaker

Line-in: 	Audio input to sound chip

MIC: 	Microphone Connector

Game/MIDI: 	For joystick or MIDI Device

(8)	Serial Port COM1: COM1

COM1 is a 9-pin D-Subminiature male connector. The On-board serial port can be set disable through BIOS SETUP. Please refer to Chapter 3 “INTEGRATED PERIPHERALS SETUP” section for more information.

(9)	Floppy drive Connector (34-pin block): FDD

This connector supports the provided floppy drive ribbon cable. After connecting the single plug end to motherboard, connect the two plugs at other end to the floppy drives.

(10)	Primary IDE Connector (40-pin block): IDE1

This connector supports the provided IDE hard disk ribbon cable. After connecting the single plug end to motherboard, connect the two plugs at other end to your hard disk(s). If you install two hard disks, you must configure the second drive to Slave mode by setting its jumpers accordingly. Please refer to the documentation of your hard disk for the jumper settings.

(11) Secondary IDE Connector (40-pin block): IDE2

This connector connects to the next set of Master and Slave hard disks. Follow the same procedure described for the primary IDE connector. You may also configure two hard disks to be both Masters using one ribbon cable on the Primary IDE connector and another ribbon cable on the secondary IDE connector.

Two hard disks can be connected to each connector. The first HDD is referred to be the “Master” and the second HDD is referred to be the “Slave”.

For performance issues, we strongly suggest you don’t install a CD-ROM or DVD-ROM drive on the same IDE channel as a hard disk. Otherwise, the system performance on this channel may drop.

2-6-2  Headers

(1)	COM2 Headers (10-pin block): COM2 (Optional)

This board has another serial port COM2.

(2)	USB Port Headers (10-pin block): USB2

These headers are used for connecting the additional USB port plug.  By attaching an optional USB cable, your can be provided with two additional USB plugs affixed to the back panel.

(3)	IDE Activity LED: IDE LED

This connector connects to the hard disk activity indicator light on the case.

(4)	Reset switch lead: RST

This 2-pin connector connects to the case-mounted reset switch for rebooting your computer without having to turn off your power switch.  This is a preferred method of rebooting in order to prolong the lift of the system’s power supply.  See the figure below.

(5)	Keyboard lock switch: KB LOCK

This 2-pin connector connects to the case-mounted key switch for locking the keyboard for security purposes.

(6)	Speaker connector: SPKR

This 4-pin connector connects to the case-mounted speaker. See the figure below.

(7)	Power LED: PWR LED

The Power LED is light on while the system power is on.  Connect the Power LED from the system case to this pin.

(8)	Power switch: PWR BTN

This 2-pin connector connects to the case-mounted power switch to power ON/OFF the system.

(9)	Wake On-LAN Headers (3-pin):  WOL1

This connector connects to a LAN card with a WAKE ON-LAN output. This connector power up the system when a wake up signal is received through the LAN card.

NOTE:	This feature requires that Wake On LAN or Ring In Wake up is enabled.

(10)	FAN Speed Headers (3-pin): CPUFAN, SYSFAN

These connectors support cooling fans of 350mA (4.2 Watts) or less, depending on the fan manufacturer, the wire and plug may be different.  The red wire should be positive, while the black should be ground.  Connect the fan’s plug to the board taking into consideration the polarity of connector.

(11)	IR infrared module Headers (10-pin): IR, CIR

This connector supports the optional wireless transmitting and receiving infrared module.  You must configure the setting through the BIOS setup to use the IR function.

(12)	CD Audio-In Headers (4-pin): CDIN1, CDIN2

CDIN1 and CDIN2 are the connectors for CD-Audio Input signal. Please connect it to CD-ROM CD-Audio output connector.

# 2-7	 Starting Up Your Computer

- After all connections are made, close your computer case cover.

- Be sure all the switches are off, and check that the power supply input voltage is set to proper position, usually in-put voltage is 220V240V or 110V120V depending on your country’s voltage used.

- Connect the power supply cord into the power supply located on the back of your system case according to the user’s manual.

- Turn on your peripherals in following order:

    - Your monitor.

    - Other external peripheral (Printer, Scanner, External Modem etc…)

    - Your system power.  For ATX power supplies, you need to turn on the power supply and press the ATX power switch on the front side of the case.

- The power LED on the front panel of the system case will light on.  The LED on the monitor may light up or switch between orange and green after the system is on if it complies with green standards or if it is has a power standby feature.  The system will then run power-on test.  While the test is running, the BIOS will alarm beeps or additional message will appear on the screen.

If you do not see any thing within 30 seconds from the time you turn on the power, the system may have failed on power-on test. Recheck your jumper settings and connections or call your retailer for assistance.

| Beep | Meaning |
| --- | --- |
| One short beep when displaying logo | No error during POST |
| Long beeps in an endless loop | No DRAM install or detected |
| One long beep followed by three short beeps | Video card not found or video card memory bad |
| High frequency beeps when system is working | CPU overheated System running at a lower frequency |

- During power-on, press <Delete> key to enter BIOS setup.  Follow the instructions in Chapter 3.

- Power off your computer: You must first exit or shut down your operating system before switch off the power switch.  For ATX power supply, you can press ATX power switching after exiting or shutting down your operating system.  If you use Windows 9X, click “Start” button, click “Shut down” and then click “Shut down the computer?” The power supply should turn off after windows shut down.

## Chapter 3

Introducing BIOS

The BIOS is a program located on a Flash Memory on the motherboard.  This program is a bridge between motherboard and operating system.  When you start the computer, the BIOS program gain control.  The BIOS first operates an auto-diagnostic test called POST (power on self test) for all the necessary hardware, it detects the entire hardware device and configures the parameters of the hardware synchronization.  Only when these tasks are completely done it gives up control of the computer to operating system (OS).  Since the BIOS is the only channel for hardware and software to communicate, it is the key factor for system stability, and in ensuring that your system performance as its best.

In the BIOS Setup main menu of Figure 3-1, you can see several options.  We will explain these options step by step in the following pages of this chapter, but let us first see a short description of the function keys you may use here:

- Press <Esc> to quit the BIOS Setup.

- Press  (up, down, left, right) to choose, in the main menu, the option you want to confirm or to modify.

- Press <F10> when you have completed the setup of BIOS parameters to save these parameters and to exit the BIOS Setup menu.

- Press Page Up/Page Down or +/– keys when you want to modify the BIOS parameters for the active option.

# 3-1	 Entering Setup

Power on the computer and by pressing <Del> immediately allows you to enter Setup.

If the message disappears before your respond and you still wish to enter Setup, restart the system to try again by turning it OFF then ON or pressing the “RESET” button on the system case. You may also restart by simultaneously pressing <Ctrl>, <Alt> and <Delete> keys.  If you do not press the keys at the correct time and the system does not boot, an error message will be displayed and you will again be asked to

Press <F1> to continue, <Ctrl-Alt-Esc> or <Del> to enter Setup

# 3-2	 Getting Help

Main Menu

The on-line description of the highlighted setup function is displayed at the bottom of the screen.

Status Page Setup Menu/Option Page Setup Menu

Press <F1> to pop up a small help window that describes the appropriate keys to use and the possible selections for the highlighted item.  To exit the Help Window, press <Esc>.

# 3-3	 The Main Menu

Once you enter Award BIOS CMOS Setup Utility, the Main Menu (Figure 3-1) will appear on the screen.  The Main Menu allows you to select from fourteen setup functions and two exit choices.  Use arrow keys to select among the items and press <Enter> to accept or enter the sub-menu.

| CMOS Setup Utility – Copyright(C) 1984-2000 Award Software |  |
| --- | --- |
| Standard CMOS Features Advanced BIOS Features Advanced Chipset Features Integrated Peripherals Power Management Setup PnP/PCI Configurations PC Health Status | Frequency/Voltage Control Load optimized Defaults Load Standard Defaults Set Supervisor Password Set User Password Save & Exit Setup Exit Without Saving |
| Esc : Quit ↑↓→ ← : Select Item F10 : Save & Exit Setup |  |
| Time, Date, Hard Disk Type... |  |

Figure 3-1

Standard CMOS Features

Use this Menu for basic system configurations.

Advanced BIOS Features

Use this menu to set the Advanced Features available on your system.

Advanced Chipset Features

Use this menu to change the values in the chipset registers and optimize your system’s performance.

Integrated Peripherals

Use this menu to specify your settings for integrated peripherals.

Power Management Setup

Use this menu to specify your settings for power management.

PnP/PCI configurations

This entry appears if your system supports PnP/PCI.

PC Health Status

This entry shows your PC health status.

Frequency/Voltage Control

Use this menu to specify your settings for frequency/voltage control.

Load Optimized Defaults

Use this menu to load the BIOS default values that are factory settings for optimal system performances.

Load Standard Defaults

Use this menu to load the BIOS default values for the minimal/stable system performance.

Set Supervisor/User Password

Use this menu to set User and Supervisor Passwords.

Save & Exit Setup

Save CMOS value changes to CMOS and exit setup.

Exit Without Saving

Abandon all CMOS value changes and exit setup.

# 3-4	 Standard CMOS Features

The items in Standard CMOS Setup Menu are divided into several categories.  Each category includes no, one or more than one setup items.  Use the arrow keys to highlight the item and then use the <PgUp> or <PgDn> keys to select the value you want in each item.

| CMOS Setup Utility – Copyright(C) 1984-2000 Award Software Standard CMOS Features |  |
| --- | --- |
| Date (mm:dd:yy) Wed, Jul, 26 2000 Time (hh:mm:ss) 17 : 35 : 22 > IDE Primary Master Press Enter None > IDE Primary Slave Press Enter None > IDE Secondary Master Press Enter None > IDE Secondary Slave Press Enter None Drive A 1.44M, 3.25 in. Drive B None Video EGA/VGA Halt On All,But Keyboard Base Memory 640K Extended Memory 56320K Total Memory 57344K | Item Help |
|  | Menu Level > Change the day, month, year and century |
| ↑↓→← Move Enter:Select Item +/-/PU/PD:Value F10:Save ESC:Exit F1:General Help F5:Previous Values F6:Optimized Defaults F7:Standard Defaults |  |

Date

The date format is <day><month><date><year>.

Day	  Day of the week, from Sun to Sat, determined by BIOS. Read-only.

Month	  The month from Jan. through Dec.

Date	  The date from 1 to 31 can be keyed by numeric function keys.

Year	  The year depends on the year of the BIOS.

Time

The time format is <hour><minute><second>.

Primary Master/Primary Slave

Secondary Master/Secondary Slave

Press PgUp/<+> or PgDn/<–> to select Manual, None, Auto type.  Note that the specifications of your drive must match with the drive table.  The hard disk will not work properly if you enter improper information for this category.  If your hard disk drive type is not matched or listed, you can use Manual to define your own drive type manually.

If you select Manual, related information is asked to be entered to the following items. Enter the information directly from the keyboard.  This information should be provided in the documentation from your hard disk vendor or the system manufacturer.

If the controller of HDD interface is SCSI, the selection shall be “None”.

If the controller of HDD interface is CD-ROM, the selection shall be “None”

Access Mode	The settings are Auto Normal, Large, and LBA.

Cylinder		number of cylinders

Head		number of heads

Precomp		write precomp

Landing Zone	landing zone

Sector		number of sectors

# 3-5	 Advanced BIOS Features

| CMOS Setup Utility – Copyright(C) 1984-2000 Award Software Advanced BIOS Features |  |
| --- | --- |
| Virus Warning Disabled CPU Internal Cache Enabled External Cache Enabled CPU L2 Cache ECC Checking Disabled Processor Number Feature Enabled Quick Power On Self Test Enabled First Boot Device Floppy Second Boot Device HDD-0 Third Boot Device LS120 Fourth Boot Device Disabled Swap Floppy Drive Disabled Boot Up Floppy Seek Enabled Boot Up NumLock Status Off Gate A20 Option Fast Typematic Rate Setting Disabled Typematic Rate (Chars/Sec) 6 Typematic Delay (Msec) 250 Security Option Setup OS Select For DRAM > 64MB Non-OS2 | Item Help |
|  | Menu Level > Allows you to choose The VIRUS warning feature for IDE Hard Disk boot sector protection. If this function is enabled and someone attempt to write data into this area, BIOS will show a warning message on screen and alarm beep |
| ↑↓→← Move Enter:Select Item +/-/PU/PD:Value F10:Save ESC:Exit F1:General Help F5:Previous Values F6:Optimized Defaults F7:Standard Defaults |  |

Virus Warning

It allows you to choose the VIRUS Warning feature for IDE Hard Disk boot sector protection.  If this function is enabled and someone attempt to write data into this area, BIOS will show a warning message on screen and alarm beep.

Disabled (default)	No warning message to appear when anything attempts to access the boot sector or hard disk partition table.

Enabled	Activates automatically when the system boots up causing a warning message to appear when anything attempts to access the boot sector of hard disk partition table.

CPU Internal Cache

The default value is Enabled.

Enabled (default)	Enable cache

Disabled		Disable cache

Note:	  The internal cache is built in the processor.

External Cache

Choose Enabled or Disabled.  This option enables the Level 2 cache memory.

CPU L2 Cache ECC Checking

Choose Enabled or Disabled.  This option enables the Level 2 cache memory ECC (error check correction).

Processor Number Feature

This option is for Pentium III processor.  During Enabled, this will check the CPU Serial number.  Disabled this option if you don’t want the system to know the Serial number.

Quick Power On Self-Test

This category speeds up Power On Self Test (POST) after you power on the computer.  If this is set to Enable, BIOS will shorten or skip some check items during POST.

Enabled (default)	Enable quick POST

Disabled		Normal POST

First/Second/Third/Fourth Boot Device

The BIOS attempts to load the operating system from the devices in the sequence selected in these items. The settings are Floppy, LS/ZIP, HDD-0/HDD-1/HDD-3, SCSI, CDROM, LAN and Disabled.

Swap Floppy Drive

Switches the floppy disk drives between being designated as A and B.  Default is Disable.

Boot Up Floppy Seek

During POST, BIOS will determine if the floppy disk drive installed is 40 or 80 tracks.  360K type is 40-track while 760K, 1.2M and 1.44M are all 80-track.

Boot Up NumLock Status

The default value is On.

On (default)	  Keypad is numeric keys.

Off		  Keypad is arrow keys.

Gate A20 Option

Normal		  The A20 signal is controlled by keyboard controller or chipset hardware.

Fast (default)	  The A20 signal is controlled by port 92 or chipset specific method.

Typematic Rate Setting

Keystrokes repeat at a rate determined by the keyboard controller. When enabled, the typematic rate and typematic delay can be selected.  The settings are: Enabled/Disabled.

Typematic Rate (Chars/Sec)

Sets the number of times a second to repeat a keystroke when you hold the key down.  The settings are: 6, 8, 10, 12, 15, 20, 24, and 30.

Typematic Delay (Msec)

Sets the delay time after the key is held down before is begins to repeat the keystroke.  The settings are 250, 500, 750, and 1000.

Security Option

This category allows you to limit access to the system and Setup, or just to Setup.

System	The system will not boot and access to Setup will be denied if the correct password is not entered at the prompt.

Setup (default)	The system will boot, but access to Setup will be denied if the correct password is not entered prompt.

OS Select For DRAM > 64MB

Allows OS2 to be used with >64MB or DRAM. Settings are Non-OS/2 (default) and OS2.  Set to OS/2 if using more than 64MB and running OS/2.

Report No FDD For Win 95

Whether report no FDD for Win 95 or not.  The settings are: Yes, No.

# 3-6	 Advanced Chipset Features

The Advanced Chipset Features Setup option is used to change the values of the chipset registers.  These registers control most of the system options in the computer.

| CMOS Setup Utility – Copyright(C) 1984-2000 Award Software Advanced Chipset Features |  |
| --- | --- |
| Advanced DRAM Control 1 Press Enter System BIOS Cacheable Enabled Video RAM Cacheable Enabled Memory Hole at 15M-16M Disabled AGP Aperture Size 64MB Graphic Window WR Combin Enabled Concurrent function(MEM) Enabled Concurrent function(PCI) Enabled CPU Pipeline Control Enabled PCI Delay Transaction Disabled Memory Parity Check Disabled | Item Help |
|  | Menu Level > |
| ↑↓→← Move Enter:Select Item +/-/PU/PD:Value F10:Save ESC:Exit F1:General Help F5:Previous Values F6:Optimized Defaults F7:Standard Defaults |  |

Advanced DRAM Control 1

Please refer to section 3-6-1

System BIOS Cacheable

Selecting Enabled allows caching of the system BIOS ROM at F0000h-FFFFFh, resulting in better system performance.  However, if any program writes to this memory area, a system error may result.  The settings are: Enabled and Disabled.

Video RAM Cacheable

Select Enabled allows caching of the video BIOS, resulting in better system performance.  However, if any program writes to this memory area, a system error may result. The settings are: Enabled and Disabled.

Memory Hole at 15M-16M

You can reserve this area of system memory for ISA adapter ROM.  When this area is reserved, it cannot be cached.  The user information of peripherals that need to use this area of system memory usually discusses their memory requirements. The settings are: Enabled and Disabled.

PCI Delay Transaction

The chipset has an embedded 32-bit posted write buffer to support delay transactions cycles.  Select Enabled to support compliance with PCI specification version 2.1.  The settings are: Enabled and Disabled.

Memory Parity Check

This function provides parity check of memory.

The choice is either Disabled or Enabled.

3-6-1   Advanced DRAM Control

| CMOS Setup Utility – Copyright(C) 1984-2000 Award Software Advanced DRAM Control 1 |  |
| --- | --- |
| Auto Configuration Auto SDRAM RAS Active Time 5T SDRAM RAS Precharg Time 3T RAS to CAS Delay 4T Dram Background Command Delay 1T LD-Off Dram RD/WR Cycles Delay 1T Write Recovery Time 2T VCM ACCT – ACT/REF Delay 9T Early CKE Delay 1T Cntrl Normal Early CKE Delay Adjust 7ns Mem Command Output Time Delay 1T SDRAM/VCM CAS Latency 3T | Item Help |
|  | Menu Level >> |
| ↑↓→← Move Enter:Select Item +/-/PU/PD:Value F10:Save ESC:Exit F1:General Help F5:Previous Values F6:Optimized Defaults F7:Standard Defaults |  |

# 3-7	 Integrated Peripherals

| CMOS Setup Utility – Copyright(C) 1984-2000 Award Software Integrated Peripherals |  |
| --- | --- |
| > OnChip IDE Function Press Enter > OnChip Device Function Press Enter > Winbond SuperIO Device Press Enter Init Display First PCI Slot System Share Memory Size 8 MB | Item Help |
|  | Menu Level > |
| ↑↓→← Move Enter:Select Item +/-/PU/PD:Value F10:Save ESC:Exit F1:General Help F5:Previous Values F6:Optimized Defaults F7:Standard Defaults |  |

OnChip IDE Function

Please refer to section 3-7-1

OnChip Device Function

Please refer to section 3-7-2

Winbond SuperIO Device

Please refer to section 3-7-3

Init Display First

This item allows you to decide to activate whether PCI Slot or AGP VGA first.  The settings are: PCI Slot, AGP Slot.

3-7-1   OnChip IDE Function

| CMOS Setup Utility – Copyright(C) 1984-2000 Award Software OnChip IDE Function |  |
| --- | --- |
| Internal PCI/IDE Both Primary Master PIO Auto Primary Slave PIO Auto Secondary Master PIO Auto Secondary Slave PIO Auto Primary Master UDMA Auto Primary Slave UDMA Auto Secondary Master UDMA Auto Secondary Slave UDMA Auto IDE Burst Mode Enabled IDE HDD Block Mode Enabled IDE Prefetch Mode Enabled | Item Help |
|  | Menu Level >> |
| ↑↓→← Move Enter:Select Item +/-/PU/PD:Value F10:Save ESC:Exit F1:General Help F5:Previous Values F6:Optimized Defaults F7:Standard Defaults |  |

Internal PCI/IDE

The integrated peripheral controller contains an IDE interface with support for two IDE channels.  Select Both to activate each channel separately.  The settings are: Both, IDE0, IDE1.

Primary/Secondary Master/Slave PIO

The four IDE PIO (Programmed Input/Output) fields let you set a PIO mode (0-4) for each of the four IDE devices that the onboard IDE interface supports.  Modes 0 through 4 provide successively increased performance. In Auto mode, the system automatically determines the best mode for each device.  The settings are: Auto, Mode 0, Mode 1, Mode 2, Mode 3, Mode 4.

Primary/Secondary Master/Slave UDMA

Ultra DMA/33 implementation is possible only if your IDE hard drive supports it and the operating environment includes a DMA driver (Windows 95 OSR2 or a third-party IDE bus master driver).  If your hard drive and your system software both support Ultra DMA/33 and Ultra DMA/66, select Auto to enable BIOS support.  The settings are: Auto, Disabled.

IDE HDD Block Mode

Block mode is also called block transfer, multiple commands, or multiple sector read/write.  If your IDE hard drive supports block mode (most new drives do), select Enabled for automatic detection of the optimal number of block read/writes per sector the drive can support.  The settings are: Enabled, Disabled.

3-7-2   OnChip Device Function

| CMOS Setup Utility – Copyright(C) 1984-2000 Award Software OnChip Device Function |  |
| --- | --- |
| AC97 Sound Device Enabled Game Port Address 201 Midi Port Address 330 Midi Port IRQ 10 AMR Modem DEVICE Enabled ETHERNET Function Enabled ETHERNET Address ID Input Press Enter Current Mac address is 003018-000001 USB Controller Enabled USB Keyboard Support Disabled | Item Help |
|  | Menu Level >> |
| ↑↓→← Move Enter:Select Item +/-/PU/PD:Value F10:Save ESC:Exit F1:General Help F5:Previous Values F6:Optimized Defaults F7:Standard Defaults |  |

AC97 Sound Device

This item allows you to decide to enable/disable the chipset family to support AC97 Audio.  The settings are: Enabled, Disabled.

Game Port Address/Midi Port Address

This will determine which Address the Game Port/Midi Port will use.

AMR Modem DEVICE

This item allows you to decide to enable/disable the chipset family to support AC97 Modem.  The settings are: Auto, Disabled.

USB Controller

Select Enabled if your system contains a Universal Serial Bus (USB) controller and you have a USB peripherals.  The settings are: Enabled, Disabled.

USB Keyboard Support

Select Enabled if your system contains a Universal Serial Bus (USB) controller and you have a USB keyboard.  The settings are: Enabled, Disabled.

ETHERNET Address ID Input

This item allows you to setting the Mac address from 030018 to 000001.

3-7-3   Winbond SuperIO Device

| CMOS Setup Utility – Copyright(C) 1984-2000 Award Software Winbond SuperIO Device |  |
| --- | --- |
| Onboard FDD Controller Enabled Onboard Serial Port 1 3F8/IRQ4 Onboard Serial Port 2 2F8/IRQ3 UART 2 Mode Normal RxD,TxD Active Hi,Hi IR Transmission Delay Enabled IR Duplex Mode Half Use IR Pins IR-Rx2Tx2 Onboard Parallel Port 378/IRQ7 Parallel Port Mode SPP EPP Mode Select EPP1.7 ECP Mode Use DMA 3 | Item Help |
|  | Menu Level >> |
| ↑↓→← Move Enter:Select Item +/-/PU/PD:Value F10:Save ESC:Exit F1:General Help F5:Previous Values F6:Optimized Defaults F7:Standard Defaults |  |

Onboard FDD Controller

Select Enabled if your system has a floppy disk controller (FDD) installed on the system board and you wish to use it.  If you install add-on FDC or the system has no floppy drive, select Disabled in this field.  The settings are: Enabled and Disabled.

Onboard Serial Port 1/Port 2

Select an address and corresponding interrupt for the first and the second serial ports.  The settings are: 3F8/IRQ4, 2E8/IRQ3, 3E8/IRQ4, 2F8/IRQ3, Disabled, Auto.

UART 2 Mode

This item allows you to determine which InfraRed(IR) function of the onboard I/O chip, this functions uses.

Onboard Parallel Port

There is a built-in parallel port on the on-board Super I/O chipset that Provides Standard, ECP, and EPP features. It has the following option:

Disabled

(3BCH/IRQ7)/	Line Printer port 0

(278H/IRQ5)/ 	Line Printer port 2

(378H/IRQ7) 	Line Printer port 1

Parallel Port Mode

SPP 	: Standard Parallel Port

EPP 	: Enhanced Parallel Port

ECP	: Extended Capability Port

SPP/EPP/ECP/ECP+EPP

To operate the onboard parallel port as Standard Parallel Port only, choose “SPP.”  To operate the onboard parallel port in the EPP modes simultaneously, choose “EPP.”  By choosing “ECP”, the onboard parallel port will operate in ECP mode only.  Choosing “ECP+EPP” will allow the onboard parallel port to support both the ECP and EPP modes simultaneously.  The ECP mode has to use the DMA channel, so choose the onboard parallel port with the ECP feature.  After selecting it, the following message will appear: “ECP Mode Use DMA” at this time, the user can choose between DMA channels 3 to 1.  The onboard parallel port is EPP Spec. compliant, so after the user chooses the onboard parallel port with the EPP function, the following message will be displayed on the screen: “EPP Mode Select.”  At this time either EPP 1.7 spec. or EPP 1.9 spec. can be chosen.

# 3-8	 Power Management Setup

The Power Management Setup allows you to configure your system to most effectively save energy saving while operating in a manner consistent with your own style of computer use.

| CMOS Setup Utility – Copyright(C) 1984-2000 Award Software Power Management Setup |  |
| --- | --- |
| ACPI Function Enabled Video Off Option Susp, Stby -> off Video Off Method V/H SYNC+Blank Switch Function Break/Wake MODEM Use IRQ 3 Hot key Function As Disabled HDD Off After Disabled Power Button Function Instant-Off > PM Wake Up Events Press Enter | Item Help |
|  | Menu Level > |
| ↑↓→← Move Enter:Select Item +/-/PU/PD:Value F10:Save ESC:Exit F1:General Help F5:Previous Values F6:Optimized Defaults F7:Standard Defaults |  |

ACPI Function

This item allows you to Enabled/Disabled the Advanced Configuration and Power Management (ACPI).  The settings are Enabled and Disabled.

Video Off Option

This determines the manner in which the monitor is blanked.  The choice are Suspend  off, All Modes  Off, and Always On.

Video Off Method

This determines the manner in which the monitor is blanked.

DPMS (default)	Initial display power management signaling.

Blank Screen	This option only writes blanks to the video buffer.

V/H SYNC+Blank	This selection will cause the system to turn off the vertical and horizontal synchronization ports and write blanks to the video buffer.

Modem Use IRQ

This determines the IRQ in which the MODEM can use.

The settings are: 3, 4, 5, 7, 9, 10, 11, NA.

Power Button Function

Pressing the power button for more than 4 seconds forces the system to enter the Soft-Off state.  The settings are: Delay 4 Sec, Instant-Off.

PM Wake-Up Events

Please refer to section 3-8-1

3-8-1   PM Wake up Events

| CMOS Setup Utility – Copyright(C) 1984-2000 Award Software PM Wake Up Events |  |
| --- | --- |
| IRQ [3-7,9-15],NMI Enabled IRQ 8 Break Suspend Disabled Ring Power Up Control Disabled MACPME Power Up Control Disabled PCIPME Power Up Control Disabled KB Power ON Password Enter Power Up by Alarm Disabled x Month Alarm NA x Day of Month Alarm 0 x Time (hh:mm:ss) Alarm 0 : 0 : 0 | Item Help |
|  | Menu Level >> |
| ↑↓→← Move Enter:Select Item +/-/PU/PD:Value F10:Save ESC:Exit F1:General Help F5:Previous Values F6:Optimized Defaults F7:Standard Defaults |  |

Ring Power Up Control

During Disabled, the system will ignore any incoming call from the modem. During Enabled, the system will boot up if there’s an incoming call from the modem.

MACPME Power Up Control

During Disabled, the system will ignore any incoming call from the EtherNet controller. During Enabled, the system will boot up if there’s an incoming call from the EtherNet controller.

PCIPME Power Up Control

This will enable the system to wake up by PCI device Power Management function.

The settings are: Enabled and Disabled.

KB Power ON Password

This item can setting Power On Password, if you Enabled keyboard Power On function then you can Power On system by key-in the password which you setting.

Power Up by Alarm

This function is for setting date and time for your computer to boot up.  During Disabled, you cannot use this function.  During Enabled, choose the Date and Time Alarm:

Date(of month) Alarm

You can choose which month the system will boot up.  Set to 0, to boot every day.

Time(hh:mm:ss) Alarm

You can choose what hour, minute and second the system will boot up.

Note:	If you have change the setting, you must let the system boot up until it goes to the operating system, before this function will work.

# 3-9	 PnP/PCI Configuration Setup

This section describes configuring the PCI bus system. PCI, or Personal Computer Interconnect, is a system which allows I/O devices to operate at speeds nearing the speed the CPU itself uses when communicating with its own special components. This section covers some very technical items and it is strongly recommended that only experienced users should make any changes to the default settings.

| CMOS Setup Utility – Copyright(C) 1984-2000 Award Software PnP/PCI Configurations |  |
| --- | --- |
| Reset Configuration Data Disabled Resources Controlled By Manual > IRQ Resources Press Enter PCI/VGA Palette Snoop Disabled | Item Help |
|  | Menu Level > Default is Disabled. Select Enabled to Reset Extended System Configuration Data (ESCD) when you exit Setup if you have Installed a new add-on and the system reconfiguration has caused such a serious conflict that the OS cannot boot |
| ↑↓→← Move Enter:Select Item +/-/PU/PD:Value F10:Save ESC:Exit F1:General Help F5:Previous Values F6:Optimized Defaults F7:Standard Defaults |  |

Reset Configuration Data

Normally, you leave this field Disabled. Select Enabled to reset Extended System Configuration Data (ESCD) when you exit Setup if you have installed a new add-on and the system reconfiguration has caused such a serious conflict that the operating system can not boot.  The settings are: Enabled and Disabled.

Resource Controlled By

The Award Plug and Play BIOS has the capacity to automatically configure all of the boot and Plug and Play compatible devices.  However, this capability means absolutely nothing unless you are using a Plug and Play operating system such as Windows95/98.  If you set this field to “manual” choose specific resources by going into each of the sub menu that follows this field (a sub menu is preceded by a “>”).

The settings are: Auto(ESCD), Manual.

IRQ Resources

When resources are controlled manually, assign each system interrupt a type, depending on the type of device using the interrupt.

Please refer to section 3-9-1

PCI/VGA Palette Snoop

Leave this field at Disabled.  The settings are Enabled, Disabled.

3-9-1   IRQ Resources

| CMOS Setup Utility – Copyright(C) 1984-2000 Award Software IRQ Resources |  |
| --- | --- |
| IRQ-3 assigned to PCI Device IRQ-4 assigned to PCI Device IRQ-5 assigned to PCI Device IRQ-7 assigned to PCI Device IRQ-9 assigned to PCI Device IRQ-10 assigned to PCI Device IRQ-11 assigned to PCI Device IRQ-12 assigned to PCI Device IRQ-14 assigned to PCI Device IRQ-15 assigned to PCI Device | Item Help |
|  | Menu Level > Default is Disabled. Select Enabled to Reset Extended System Configuration Data (ESCD) when you exit Setup if you have Installed a new add-on and the system reconfiguration has caused such a serious conflict that the OS cannot boot |
| ↑↓→← Move Enter:Select Item +/-/PU/PD:Value F10:Save ESC:Exit F1:General Help F5:Previous Values F6:Optimized Defaults F7:Standard Defaults |  |

# 3-10  PC Health Status

This section shows the Status of you CPU, Fan, Warning for overall system status.  This is only available if there is Hardware Monitor onboard.

| CMOS Setup Utility – Copyright(C) 1984-2000 Award Software PC Health Status |  |
| --- | --- |
| CPU Warning Temperature Disabled Shutdown Temperature Disabled Show PC Health in Post Enabled Current System Temp. 28C/82F Current CPU Temperature 29C/84F Current CPUFAN Speed 4687 RPM Current SYSFAN Speed 0 RPM Vcore 1.69V Vcc3.3 3.31V + 5V 5.00V +12V + 12.02V -12V - 12.16V 5VSB(V) 5.45V | Item Help |
|  | Menu Level > |
| ↑↓→← Move Enter:Select Item +/-/PU/PD:Value F10:Save ESC:Exit F1:General Help F5:Previous Values F6:Optimized Defaults F7:Standard Defaults |  |

Show PC Health in Post

During Enabled, it displays information list below.  The choice is either Enabled or Disabled

Current CPU Temperature/Current System Temp/Current CPUFAN, SYSFAN Speed/ Vcore/ Vcc/3.3V/+5V/+12V/-12V/5VSB(V)

This will show the CPU/FAN/System voltage chart and FAN Speed.

# 3-11  Frequency/Voltage Control

This section is for setting CPU Frequency Control.

| CMOS Setup Utility – Copyright(C) 1984-2000 Award Software Frequency/Voltage Control |  |
| --- | --- |
| Cyrix III Clock Ratio Default Auto Detect DIMM/PCI Clk Enabled Spread Spectrum Disabled CPU Host/DRAM Clock Default CPU Clock Ratio Jumpless x 3.0 | Item Help |
|  | Menu Level > CyrixIII CPU Ratio Adjust |
| ↑↓→← Move Enter:Select Item +/-/PU/PD:Value F10:Save ESC:Exit F1:General Help F5:Previous Values F6:Optimized Defaults F7:Standard Defaults |  |

Cyrix III Clock Ratio

This item allows you to set Cyrix III Clock Ratio.   The settings are Default, X3.5, X3, X4, X4.5, X5, X5.5, X6.

Auto Detect DIMM/PCI Clk

This item allows you to enable/disable auto detect DIMM/PCI Clock.

The settings are: Enabled, Disabled.

CPU Host/DRAM Clock

This item allows you to set the CPU Host/DRAM Clock

The choice are: 	Default, 66/66Mhz, 66/100Mhz, 75/75MHz, 83/83MHz, 95/95MHz, 96/96MHz, 97/97MHz, 100/100MHz, 100/133MHz, 112/112MHz, 133/100MHz, 133/133MHz

# 3-12  Load Standard/Optimized Defaults

Load Standard Defaults

When you press <Enter> on this item, you get confirmation dialog box with a message similar to:

Load Standard Defaults (Y/N)? N

Pressing <Y> loads the BIOS default values for the most stable, minimal-performance system operations.

Load Optimized Defaults

When you press <Enter> on this item, you get a confirmation dialog box with a message similar to:

Load Optimized Defaults (Y/N)? N

Pressing <Y> loads the default values that are factory settings for optimal performance system operations.

# 3-13  Set Supervisor/User Password

You can set either supervisor or user password, or both of them.  The differences are:

Supervisor password: 	Can enter and change the options of the setup menus.

User password:	Can only enter but do not have the right to change the options of the setup menus.  When you select this function, the following message will appear at the center of the screen to assist you in creating a password.

ENTER PASSWORD:

Type the password, up to eight characters in length, and press <Enter>.  The password typed now will clear any previously entered password from CMOS memory.  You will be asked to confirm the password.  Type the password again and press <Enter>.  You may also press <Esc> to abort the selection and not enter a password.

To disable a password, just press <Enter> when you are prompted to enter the password.  A message will confirm that the password will be disabled.  Once the password is disabled, the system will boot and you can enter Setup freely.

PASSWORD DISABLED.

When a password has been enabled, you will be prompted to enter it every time you try to enter Setup.  This prevents an unauthorized person from changing any part of your system configuration.

Additionally, when a password is enabled, you can also require the BIOS to request a password every time your system is rebooted.  This would prevent unauthorized use of your computer.

You determine when the password is required within the BIOS Features Setup Menu and its Security option.  If the Security option is set to “System”, the password will be required both at boot and at entry to Setup.  If set to “Setup”, prompting only occurs when trying to enter Setup.

## Chapter 4

## DRIVER & FREE PROGRAMS INSTALLATION

Check your package and there is a CD Utility included. This CD consists of all DRIVERS you need and some free utility programs. In addition, this CD also includes an auto-install software, which will guide you through the installation.

4-1 Driver Installation

Platform: WINDOWS 95/98/98SE/ME/NT4.0/2000

Insert CD into your CD-ROM drive and the Main Menu should appear as below.

Note! If the menu does not appear, double-click MY COMPUTER/CD-ROM drive.

From MAIN MENU, select the Mainboard Driver Install and then select 630E Driver, all the drivers of 630E will appear as below. You are required to install the LAN Driver, Sound and VGA.

Note! 	If you experience any problem when installing drivers, please go to MY COMPUTER→PROPERTIES→HARDWARE MANAGEMENT and install the driver that hasn’t finished.

4-2 BIOS Install Utility & BIOS Update

By clicking the M/B Flash Utility Install in the Main Menu, you can install the BIOS easily. You can also update your BIOS at any time by downloading the BIOS files from our website www.topaction.com.tw. Please make sure to download the correct files when you do so. Failure to do so may cause serious damage to your system.

用户简易安装手册

第一步：安装CPU与散片

1．将处理器插座的固定杆手把拉起，使之呈900位置。

2．寻找CPU的缺角边缘。CPU边缘应该朝向插座上的针脚缺角端（靠近固定村手把旋转部分），CPU边缘唯有对准方向，才能安装。

3．拉下CPU插座的固定杆，完成CPU的安装。

4．安装散热器。先将散热器一端靠在CPU插座。

5．下压散热器并将散热器的另一端钩住CPU插座。

第二步：安装内存

1．将内存模组插槽两端的白色固定卡柄往外扳开。

2．内存有缺口的一边对应着内存插槽插入。所有的记忆在安装时，只能朝向同一个方向。

3．把内存牢固的插进插槽。当正确插入后，插槽两端的固定卡柄便会锁上并且固定住内存模组。

第三步：跳线设定

630E可支持所有Socket 370的CPU，包括Celeron I到Celeron III 和Pentium全系列产品。对应不同的CPU，其不同的跳线设置如下表所示。

** 如果 CPU 电压是 1.5V 就必须将 JP7 跳帽插上

| 其它 CPU | 奔腾III或赛扬CPU 1.5V 电压 |
| --- | --- |
| JP7 (Open / OFF) | JP7 (Close / ON) |

JP10, JP1, JP11 设定表

| For CPU 300~500MHz | 其他CPU |
| --- | --- |

第四步：连接IDE装置与软驱排线

1．连接软驱的排线时，需将排线中带有红颜色的一侧对准第一脚位，连接时请注意是否对准。

2．连接IDE装置的排线时,需将排线中带有红颜色的一侧对准第一脚位，连接时请注意是否对准。

第五步：安装扩充卡

1．仔细阅读扩充卡的使用说明，为扩充卡进行必要的硬体或软体设定，如跳线或开关

2．拆掉机壳盖和机箱后面的扩充卡挡片，并保存好挡片以备日后使用。

3．小心将扩充卡插入插槽中，并确认牢固。

4．将扩充卡与电脑机壳以螺丝锁牢。

5．安装外部机壳。

6．如果必要，请进入BIOS进行设置。

7．为扩充卡安装必要的驱动程序。

第六步：连接机壳面板指示灯

请将各指示灯导线接到其对应电源接头上。主板上的电源接头如图所示。

第七步：连接电源线

电源线接头必须依正确的方式插入。

第八步：连接键盘，鼠标，显示器，扬声器

键盘，鼠标，显示器和扬声器的接口位置如下图。

滑鼠 网卡 打印头 游戏杆/MIDI头

键盘 USB COM1 VGA 音效接口